DESIGN REVIEW
DOCUMENT

Project	BEE-DAL

Subsystem	OBC

Version	V1.0

Document Version	V1.0.0

Author	BAO BUI

Review Owner	BAO BUI

Date	2026-July-13

# DOCUMENT HISTORY

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| V1.0.0 | 2026-June-10 | BAO BUI | Review Round 1 |

# REVIEW SUMMARY

| Review Round | Reviewer(s) | Description | Total Issues | Closed | Open |
| --- | --- | --- | --- | --- | --- |

# REVIEW ROUND 1

### Review Information

Schematic Design Version: V1.0.0

PCB Design Version: V1.0

Review Date: 2026-June-10

Git link:

Reviewer: HIEU CAO

## Issue 1.0.0.1

| Description | Zenner/BJT bảo vệ CONN_IN_SEL |
| --- | --- |
| Location | CONNECTOR_IN.Schdoc |
| Severity |  |
| Recommendation | - Zenner/BJT bảo vệ CONN_IN_SEL |
| Owner | CAM DOAN |
| Status | CLOSED |
| Resolution Version | 1.0.1 |
| Related Change | Change 1.1 |

## Issue 1.0.0.2

| Description | 12V IN chua co eFuse |
| --- | --- |
| Location | CONNECTOR_IN.Schdoc |
| Severity |  |
| Recommendation | Thêm eFuse |
| Owner | CAM DOAN |
| Status | Closed |
| Resolution Version | 1.0.1 |
| Related Change | Change 1.2 |

## Issue 1.0.0.3

| Description | IC 74HC244 điện áp hoạt động là 5V => sai |
| --- | --- |
| Location | CONNECTOR_OUT |
| Severity |  |
| Recommendation | Dùng SN74AVC4T245 |
| Owner | CAM DOAN |
| Status | Closed |
| Resolution Version | 1.0.1 |
| Related Change | Change 1.3 |

## Issue 1.0.0.4

| Description | R31 cần DNP |
| --- | --- |
| Location |  |
| Severity |  |
| Recommendation | R31 đổi thành DNP/1M |
| Owner | CAM DOAN |
| Status | Closed |
| Resolution Version | 1.0.1 |
| Related Change | Change 1.3 |

## Issue 1.0.0.5

| Description | PROGRAM_B chưa được điều khiển |
| --- | --- |
| Location |  |
| Severity |  |
| Recommendation | Thêm điều khiển PROGRAM_B bởi MPU qua bjt đệm |
| Owner | CAM DOAN |
| Status | Closed |
| Resolution Version | 1.0.1 |
| Related Change | Change 1.3 |

## Issue 1.0.0.6

| Description | FPGA chưa có IO vào MPU, thêm 2 LED, thêm 2 button, thêm 1 uart để debug |
| --- | --- |
| Location |  |
| Severity |  |
| Recommendation | FPGA chưa có - 4 IO vào MPU, - 2 LED, - 2 button, - 1 uart để debug |
| Owner | CAM DOAN |
| Status | Closed |
| Resolution Version | 1.0.1 |
| Related Change | Change 1.3 |

## Issue 1.0.0.7

| Description | MPU và FPGA không có giao tiếp SPI |
| --- | --- |
| Location |  |
| Severity |  |
| Recommendation |  |
| Owner | CAM DOAN |
| Status | OPEN |
| Resolution Version | 1.0.1 |
| Related Change | Change 1.3 |

## Issue 1.0.0.8

| Description |  |
| --- | --- |
| Location |  |
| Severity |  |
| Recommendation | - QSPI_SCK đấu thêm 1 gpio vào clock của CC_CLOCK0 |
| Owner | CAM DOAN |
| Status | Closed |
| Resolution Version | 1.0.1 |
| Related Change | Change 1.3 |

## Issue 1.0.0.9

| Description |  |
| --- | --- |
| Location |  |
| Severity |  |
| Recommendation | VCCBAT nối 1.8V |
| Owner | CAM DOAN |
| Status | Closed |
| Resolution Version | 1.0.1 |
| Related Change | Change 1.3 |

## Issue 1.0.0.10

| Description |  |
| --- | --- |
| Location |  |
| Severity |  |
| Recommendation | PDUC_B cần pull up lên 3.3V |
| Owner | CAM DOAN |
| Status | Closed |
| Resolution Version | 1.0.1 |
| Related Change | Change 1.3 |

# DESIGN V1.0.1

### Version Information

Previous Version: V1.0.0

Current Version: V1.0.1

Purpose: Fix Review Round 1 findings

### Issue Resolution Matrix

| Change ID | Related Issue | Description | Status |
| --- | --- | --- | --- |
| Change 1.1 | Issue 1.1 | Add TVS diode | Fixed |

## Change 1.0.1

| Linked Issue | Issue 1.0.0.1 |
| --- | --- |
| Modified Documents |  |
| Description | Zenner/BJT bảo vệ CONN_IN_SEL |
| Evidence |  |

## Change 1.0.2

| Linked Issue | Issue 1.0.0.2 |
| --- | --- |
| Modified Documents |  |
| Description | Thêm eFuse |
| Evidence |  |

## Change 1.0.3

| Linked Issue | Issue 1.0.0.3 |
| --- | --- |
| Modified Documents |  |
| Description | Dùng SN74AVC4T245 |
| Evidence |  |

## Change 1.0.4

| Linked Issue | Issue 1.0.0.5 |
| --- | --- |
| Modified Documents |  |
| Description | R31 DNP |
| Evidence |  |

## Change 1.0.5

| Linked Issue | Issue 1.0.0.5 |
| --- | --- |
| Modified Documents |  |
| Description | Thêm điều khiển PROGRAM_B bởi MPU qua bjt đệm |
| Evidence |  |

## Change 1.0.6

| Linked Issue | Issue 1.0.0.6 |
| --- | --- |
| Modified Documents |  |
| Description | FPGA chưa có - 4 IO vào MPU, - 2 LED, - 2 button, - 1 uart để debug |
| Evidence | - 4 IO vào MPU, - 2 LED, - 2 button, - 1 uart để debug |

## Change 1.0.7

| Linked Issue |  |
| --- | --- |
| Modified Documents |  |
| Description |  |
| Evidence |  |

## Change 1.0.8

| Linked Issue |  |
| --- | --- |
| Modified Documents | FPGA clock nối với QSPI CLK? |
| Description |  |
| Evidence |  |

## Change 1.0.9

| Linked Issue |  |
| --- | --- |
| Modified Documents |  |
| Description | Nối VCCBAT vào 1.8V |
| Evidence |  |

## Change 1.0.10

| Linked Issue |  |
| --- | --- |
| Modified Documents |  |
| Description | PDUC_B cần pull up lên 3.3V |
| Evidence |  |

## Change 1.0.x

| Linked Issue |  |
| --- | --- |
| Modified Documents |  |
| Description |  |
| Evidence |  |

## Change 1.0.x

| Linked Issue |  |
| --- | --- |
| Modified Documents |  |
| Description |  |
| Evidence |  |

# REVIEW ROUND 2

### Review Information

Schematic Design Version: V1.0.1

PCB Design Version: V1.0

Review Date: 2026-June-10

Phase: Bring up

Reviewer: HIEU CAO

## Issue 1.0.1.1

| Description | MIPI CSI2 Camera xuyên qua nhiều board có nguy cơ tín hiệu không đảm bảo -> gây ra hiện tượng timerout trên dây, phải chỉnh nhẹ dây mới hoạt động. |
| --- | --- |
| Location | OBC ISS HW10 |
| Severity | High |
| Recommendation | Sử dụng Retimer/Redriver cho ứng dụng này. Có 2 mã thường dùng chính: SN65DPHY440SS [TI] và PI2MEQX2503A[DI]. Recommend PI2MEQX2503A vì độ đơn giản trong thiết kế. - EQ/SCL, 9dB (VIH), 10kΩ VCC 1.8V - ERC/SDA, 150ps, NC - PRE_CFG, 10dB (VIL), 10kΩ GND - VSADJ_CFG0, 250mV, NC |
| Owner | CAM DOAN |
| Status | Opened |
| Resolution Version | 1.1.0 |
| Related Change | Change |

## Issue 2.2

| Description | Power Leak EXP thông qua đường UART, 1.7V nếu UART có hoạt động nhưng không có nguồn đi vào EXP |
| --- | --- |
| Location | OBC ISS HW10 |
| Severity | Low |
| Recommendation | Sử dụng MAX232/UART Buffer để có HighZ nếu bên kia không có nguồn |
| Owner | CAM DOAN |
| Status | Opened |
| Resolution Version | 1.1.0 |
| Related Change | Change |

## Issue 2.3

| Description | OBC đang không đọc được input Do chưa nhận slow clock Trở 1M ở mọi board dùng 32.768 đều |
| --- | --- |
| Location | OBC ISS HW10 |
| Severity | Low |
| Recommendation |  |
| Owner | CAM DOAN |
| Status | Opened |
| Resolution Version | 1.1.0 |
| Related Change | Change |

Check điện trở 1M

Debounce không chạy, bỏ debounce mới nhận --> slow clock lỗi

## Issue 2.4

| Description | Power Leak EXP thông qua đường UART, 1.7V nếu UART có hoạt động nhưng không có nguồn đi vào EXP |
| --- | --- |
| Location | EXP ISS HW10 |
| Severity | Low |
| Recommendation | Sử dụng MAX232/UART Buffer để có HighZ nếu bên kia không có nguồn |
| Owner | HUY HUNG |
| Status | Opened |
| Resolution Version | 1.0 |
| Related Change | Change |

## Issue 2.5

| Description | MCU PB12 cần bắt buộc Pull Down |
| --- | --- |
| Location | EXP ISS HW10 |
| Severity | Low |
| Recommendation | Hiện tại PB12 đang Float, cần pull down trở 10k |
| Owner | HUY HUNG |
| Status | Opened |
| Resolution Version | 1.0 |
| Related Change | Change |

## Issue 2.6

| Description | Recommend dùng EXT Laser đấu riêng cho OBC thay vì hiện tại OBC phải command xuống EXP để bật Laser |
| --- | --- |
| Location | EXP ISS HW10 |
| Severity | Low |
| Recommendation |  |
| Owner | HUY HUNG |
| Status | Opened |
| Resolution Version | 1.0 |
| Related Change | Change |